MR-03 Baud 9600, Stop 1, Parity None, 8 bit

GAIN: radio buttons; Min &h32, Mid &h64, Strong &hFF (default Mid)

SPEED:  radio buttons; Slow &h0A, 2 &h14, 3 &h1E, 4 &h28, Fast &hFF (default Fast)

PNCH: slider; 1-10 (default 2) 1 = &h01 2 = &h02 10 = &h0A

D.BAND: radio buttons; Narrow &h01, Mid &h02, Wide &h03 (default Mid)

DUMP: radio buttons; Over &h01, Smooth &h02 (default Over)

D.FREQ: radio buttons; 5kHz &h40, 2.5kHz &h78, 1.2kHz &hFF (default 1.2kHz)

NUTRAL: radio buttons; Narrow &h82 &h7C, Mid &h88 &h78, Wide &h94 &h6C (default Mid)

V.INERTIA: radio buttons; Strong &h01, 2 &h02, 3 &h03, 4 &h04, OFF &FF (default OFF)

BAKTIM: radio buttons; 1 &h01, 2 &h05, 3 &h0A, 4 &h14, 5 &h28 (default 2)

ST.GAIN: slider; 1-255 (default 90)

TH.GAIN: slider ; 1-255 (default 60)

Length 18: D5 5A 64 FF 02 02 01 FF BC 44 88 78 FF 2C 05 5A 3C 87

Byte 1: Header D5

Byte 2: Header 5A

Byte 3: GAIN

Byte 4: SPEED

Byte 5: PNCH

Byte 6: D.BAND

Byte 7: DUMP

Byte 8: D.FREQ

Byte 9: Unknown, set to BC

Byte 10: Unknown set to 44

Byte 11: NUTRAL

Byte 12: NUTRAL

Byte 13: V.INERTIA

Byte 14: Unknown, set to 2C

Byte 15: BAKTIM

Byte 16: ST.GAIN

Byte 17: TH.GAIN

Byte 18: Checksum

AD Baud 1200, Stop 1, Parity None, 8 bit

Not all values for similar options are the same as the MR-03

GAIN: radio buttons; 1, 2, 3 &h03 (default 3)

SPEED: radio buttons; 1, 2 &h02, 3 (default 2)

PUNCH: slider; 1 – 10 (default 5) 5 = &h05

DEAD BAND: radio buttons; 1, 2, 3, 4 &h04, 5 (default 4)

DAMP: radio buttons; 1 &h01, 2 (default 1)

LIMIT: slider; 0 – 60 (default 25)

DRIVE FREQ: radio buttons; High, Mid, Low (default Mid)

Length 18: D5 5A 03 02 05 04 01 C8 40 80 32 5D 53 07 9E 04 1A 96

Byte 1: Header D5

Byte 2: Header 5A

Byte 3: GAIN

Byte 4: SPEED

Byte 5: PNCH

Byte 6: D.BAND

Byte 7: DUMP

Byte 8:

Byte 9:

Byte 10:

Byte 11:

Byte 12:

Byte 13:

Byte 14:

Byte 15:

Byte 16:

Byte 17:

Byte 18: Checksum

When trying to program it sends a C5 to read.  This is not needed.  The above packet can be used without a read.  Need to figure out other values first.

Mini-Z ASF Baud 9600, Stop 1, Parity None, 8 bit

For available options values are the same as MR-03

GAIN: radio buttons; Min &h32, Mid &h64, Strong &hFF (default Strong)

SPEED:  radio buttons; Slow &h0A, 2 &h14, 3 &h1E, 4 &h28, Fast &hFF (default Fast)

PNCH: slider; 1-10 (default 10) 1 = &h01 2 = &h02 10 = &h0A

D.BAND: radio buttons; Narrow &h01, Mid &h02, Wide &h03 (default Wide)

DUMP: radio buttons; Over &h01, Smooth &h02 (default Over)

D.FREQ: radio buttons; 5kHz &h40, 2.5kHz &h78, 1.2kHz &hFF (default 2.5kHz)

NUTRAL: radio buttons; Narrow &h82 &h7C, Mid &h88 &h78, Wide &h94 &h6C (default Mid)

V.INERTIA: radio buttons; Strong &h01, 2 &h02, 3 &h03, 4 &h04, OFF &FF (default 3)

Length 18: D5 5A FF FF 0A 03 01 78 BC 44 88 78 03 FF FF FF FF DD

Byte 1: Header D5

Byte 2: Header 5A

Byte 3: GAIN

Byte 4: SPEED

Byte 5: PNCH

Byte 6: D.BAND

Byte 7: DUMP

Byte 8: D.FREQ

Byte 9: Unknown, set to BC

Byte 10: Unknown set to 44

Byte 11: NUTRAL

Byte 12: NUTRAL

Byte 13: V.INERTIA

Byte 14: Unknown, set to FF

Byte 15: BAKTIM unused, set to FF

Byte 16: ST.GAIN unused, set to FF

Byte 17: TH.GAIN unused, set to FF

Byte 18: Checksum

dNaNo Baud 9600, Stop 1, Parity None, 8 bit

Only missing the BAKTIM option

GAIN: radio buttons; Min &h32, Mid &h64, Strong &hFF (default Strong)

SPEED:  radio buttons; Slow &h0A, 2 &h14, 3 &h1E, 4 &h28, Fast &hFF (default Fast)

PNCH: slider; 1-10 (default 10) 1 = &h01 2 = &h02 10 = &h0A

D.BAND: radio buttons; Narrow &h01, Mid &h02, Wide &h03 (default Wide)

DUMP: radio buttons; Over &h01, Smooth &h02 (default Over)

D.FREQ: radio buttons; 4.8kHz &h40, 2.3kHz &h78, 1.1kHz &hFF (default 4.8kHz)

NUTRAL: radio buttons; Narrow &h82 &h7C, Mid &h88 &h78, Wide &h94 &h6C (default Mid)

V.INERTIA: radio buttons; Strong &h01, 2 &h02, 3 &h03, 4 &h04, OFF &FF (default 3)

ST.GAIN: slider; 1-255 (default 128)

TH.GAIN: slider ; 1-255 (default 128)

Length 18: D5 5A FF FF 0A 03 01 40 BC 44 88 78 03 FF FF 80 80 A7

Byte 1: Header D5

Byte 2: Header 5A

Byte 3: GAIN

Byte 4: SPEED

Byte 5: PNCH

Byte 6: D.BAND

Byte 7: DUMP

Byte 8: D.FREQ

Byte 9: Unknown, set to BC

Byte 10: Unknown set to 44

Byte 11: NUTRAL

Byte 12: NUTRAL

Byte 13: V.INERTIA

Byte 14: Unknown, set to FF

Byte 15: BAKTIM unused, set to FF

Byte 16: ST.GAIN

Byte 17: TH.GAIN

Byte 18: Checksum

GAIN: Min, Mid, Strong (default).
Increases strength (power) of servo signal. The higher the gain the more the steering servo resists position change from outside influences.

SPEED (operational speed of the servo) Slow, 2, 3, 4, or Fast (default).
The higher the number the faster the steering servo movement.

PUNCH: 1 to 10 (default)
Reaction time of the steering servo.

D.BAND (dead band): Narrow, Mid, Wide (default) 
Narrows or expands the extent to which the steering servo does not react to outside influence. 

DUMP (damping servo stops) Over (default), or Smooth 
Smooth causes the servo to slow to a stop as it approaches the apparent intended steering position so that it does not go over or past that position. “Over” causes it to stop after it reaching the apparent intended position.

D.FREQ (motor drive frequency) 5kHz, 2.5kHz (default) or 1.2kHz
Edit: Throttle motor drive frequency. At 5kHz decreases torque but increases runtime, 1.2kHz increases torque but decreases runtime. 

NUTRAL (throttle neutral) Narrow, Mid (default), or Wide.
Width of neutral throttle trigger territory.

V.INERTIA (imaginary inertia control) Strong, 2, 3 (default), 4, or Off.
Decreases the drag brake effect. With V. Inertia turned “off” because of a Mini-Z’s light weight the motor and drive train drag can cause the car to skid to a stop when the throttle is released, especially with neo magnets and small pinions. A “Strong” V. Inertia setting simulates the roll through of a much heavier car.